Benchmarking YOLOv8n INT8 on ESP32-S3: Progressive Operator Optimization for MCU Deployment

Jaweed¹, Dihin Muriyatmoko²

¹Program Studi Informatika, Universitas Darussalam Gontor, Indonesia

²Program Studi Informatika, Universitas Darussalam Gontor, Indonesia

Email: jaweed@unida.gontor.ac.id

Abstract—This study analyzes the feasibility of deploying an INT8 quantized YOLOv8n model on the ESP32-S3 microcontroller for single-class object detection. The model was trained on a COCO 2017 person subset (1,600 train, 300 val) at 192×192 resolution and quantized via post-training quantization (PTQ) to INT8, yielding a model size of 3.08 MB with 4.41% mAP@0.5. Three-stage progressive optimization was performed: (1) baseline TFLite Micro reference kernels at 219.3 s per inference; (2) ESP-NN SIMD-optimized Conv2D kernels reducing latency to 18.0 s (12.2× speedup); (3) a custom LUT-based LOGISTIC kernel further reducing latency to 7.2 s (total 30.4× speedup). Operator profiling revealed the LOGISTIC operation consumed 62.3% of execution time before optimization. The measured tensor arena of 510 KB (98.5% below theoretical worst-case) confirms the model fits within the ESP32-S3 memory budget of 8 MB PSRAM and 8 MB flash. Estimated power consumption is 549 mW during inference. These results provide a reference benchmark for deploying YOLO-based detectors on Xtensa LX7 MCUs.

Index Terms—ESP32-S3; object detection; post-training quantization; TinyML; YOLOv8n

1. INTRODUCTION

Deep learning has enabled real-time object detection across various computing platforms [1]–[4]. However, most detection models require expensive, power-hungry GPU-class hardware. In recent years, the TinyML paradigm [5] has emerged as an alternative by running inference models on low-power microcontrollers (MCUs), enabling edge applications such as environmental monitoring, anomaly detection, and industrial automation without cloud dependency [6], [7].

The ESP32-S3 is a compelling MCU for computer vision applications: it provides dual-core Xtensa LX7 at 240 MHz, 8 MB flash, 8 MB PSRAM, camera support, and WiFi connectivity—at under $10 USD. Despite these capabilities, running deep learning models such as YOLO on the ESP32-S3 faces significant challenges: limited memory, constrained compute throughput, and operator support that is less extensive than desktop frameworks.

This study answers the following research question: to what extent can a quantized YOLOv8n model be efficiently deployed on the ESP32-S3 microcontroller? The key contributions are: (1) a systematic analysis of the training, quantization, and deployment pipeline for INT8 YOLOv8n on ESP32-S3; (2) progressive optimization driven by operator profiling—identifying the LOGISTIC bottleneck (62.3% of runtime), optimizing Conv2D via ESP-NN SIMD (12.2× speedup), and implementing a LUT-based LOGISTIC kernel (additional 2.5× speedup), yielding a total 30.4× improvement over the reference baseline; (3) comprehensive memory characterization on physical hardware; and (4) all research artifacts released openly for reproducibility.

2. RELATED WORK

Deploying CNNs on microcontrollers has been an active topic since CMSIS-NN [8] for ARM Cortex-M and TFLite Micro [9] as a cross-platform inference framework. MCUNet [6] demonstrated that MCU-optimized architectures can achieve competitive accuracy at low resolution. MicroNets [10] introduced neural architecture search (NAS) specifically for MCU constraints.

YOLO [1]–[4] remains the dominant object detection architecture, with the nano variant (YOLOv8n, 3.0M parameters) designed for resource-constrained deployment. Edge-YOLO [11] proposed edge-specific optimizations but focused on ARM and embedded GPU platforms rather than low-end MCUs.

Post-training quantization (PTQ) [12]–[14] has proven effective for reducing model size and improving inference speed. Deep Compression [15] demonstrated that combining pruning, quantization, and Huffman coding can drastically reduce model size. In the MCU domain, MobileNet [16] and MobileNetV2 [17] are commonly used as backbones due to their computational efficiency.

For the ESP32, ESP-NN [18] provides optimized neural network kernels using Xtensa LX7 SIMD extensions. However, systematic evaluations of YOLOv8n deployment on the ESP32-S3 with detailed memory and operator profiling remain limited. This study fills that gap.

3. METHOD

A. Dataset

We used the person subset of COCO 2017 [19], consisting of 1,599 training, 300 validation, and 100 test images. The 192×192 input resolution was chosen as the minimum resolution preserving person-level detail while minimizing computation on the ESP32-S3.

B. Model Architecture and Training

We adopted YOLOv8n [4], the nano variant with 3.0M parameters and 8.1 GFLOPs. Training proceeded in two stages: 50 epochs with a frozen backbone (transfer learning from COCO pretrained weights) followed by 50 epochs of full fine-tuning at 192×192. The AdamW optimizer was used with a learning rate of 0.001 and cosine decay. The PyTorch baseline achieved 45.59% mAP@0.5. After conversion through PyTorch→ONNX→TensorFlow→TFLite, the TFLite FP32 model achieved 12.26% mAP@0.5—a 33.33 pp drop attributable to numerical differences in the export pipeline.

C. Post-Training Quantization

Post-training quantization (PTQ) [12]–[14] was performed using TensorFlow Lite with a representative dataset of 200 images. The FP32 TFLite model (3.01 MB) was converted to INT8 (3.08 MB) with INT8 inference type and uint8 input/output tensors. The additional 7.85 pp mAP@0.5 drop from TFLite FP32 (12.26%) to INT8 (4.41%) represents a significant accuracy degradation, largely due to the low 192×192 resolution where a person occupying 10% of image height spans only 19 pixels.

D. ESP32-S3 Deployment

The firmware was developed using PlatformIO with the Arduino framework and TFLite Micro on a Seeed XIAO ESP32S3 Sense (240 MHz CPU, 8 MB flash, 8 MB PSRAM). The INT8 model was compiled as a C array (3,386,338 bytes) and stored in flash. Three kernel configurations were benchmarked progressively: (1) reference TFLite Micro kernels; (2) ESP-NN SIMD-optimized Conv2D kernels via Xtensa LX7 SIMD extensions; and (3) ESP-NN with a custom LUT-based LOGISTIC kernel.

4. RESULTS AND DISCUSSION

A. Operator Profiling and Optimization

The reference kernel configuration yielded 219,287 ms per inference—impractical for any application. ESP-NN Conv2D integration reduced latency to 18,020 ms (12.2× speedup). Operator profiling using TFLite Micro’s MicroProfiler revealed a counter-intuitive finding: the LOGISTIC (sigmoid) operation consumed 62.3% of total latency (11,758 ms), while Conv2D accounted for only 35.2% (6,634 ms). This is because LOGISTIC was not covered by ESP-NN and executed via the naive reference implementation.

We implemented an ESP-NN LOGISTIC wrapper kernel using esp_nn_logistic_s8(), which evaluates sigmoid via a 256-byte lookup table in O(1) per element. After optimization, inference latency dropped to 7,216 ms (2.5× additional speedup), yielding a total 30.4× improvement over the reference baseline. At this point, Conv2D accounts for approximately 92% of remaining latency; the LOGISTIC overhead was effectively eliminated.

B. Accuracy

The INT8 model achieved 4.41% mAP@0.5 at 192×192, down from 12.26% (TFLite FP32). At this resolution, a person occupying 10% of image height spans only 19 pixels, limiting discriminative features. The accuracy is suitable for coarse region-of-interest flagging (approximately 8 frames per minute at 7.2 s latency) but insufficient for fully autonomous detection in cluttered scenes.

C. Memory Characterization

A key finding is that the measured tensor arena was only 510 KB, dramatically lower than the theoretical worst-case estimate of 34.1 MB—a 98.5% reduction due to TFLite Micro’s memory planner reusing tensor buffers across subgraphs. Total RAM footprint (arena 510 KB + frame buffer 225 KB + stack and RTOS ~100 KB = 835 KB) fits entirely within the 8 MB PSRAM. Model weights (3.3 MB) fit within the 8 MB flash budget.

D. Power Consumption

Based on ESP32-S3 datasheet parameters at 240 MHz, the estimated power consumption during inference is 549 mW (chip) / 646 mW (USB). Full pipeline operation (inference + camera + WiFi) is estimated at 1,308 mW. A 2S 1,000 mAh battery would sustain approximately 1–2 hours of continuous inference. These figures are model-based estimates; empirical measurement using an INA219 current sensor remains future work.

E. Discussion and Limitations

The results reveal a clear accuracy–speed–size trade-off. The dominant bottleneck after optimization is Conv2D (~92% of latency), which is already running ESP-NN SIMD-optimized kernels; further gains require model architectural changes. The study has several limitations: (1) the modest 4.41% mAP@0.5 restricts autonomous operation; (2) the 7.2 s inference latency does not meet real-time requirements; (3) power figures are datasheet-based estimates; and (4) no end-to-end camera integration was performed on the physical hardware.

5. CONCLUSION

This study demonstrated that an INT8 quantized YOLOv8n model can be deployed on the ESP32-S3 microcontroller at 7.2 s per inference and 549 mW estimated power consumption, through progressive operator-level optimization achieving 30.4× total speedup over the reference baseline. The measured tensor arena of 510 KB confirms memory feasibility. A key finding—the LOGISTIC operator consuming 62.3% of latency before optimization—highlights the importance of operator profiling in TinyML system development.

Future work includes: (i) exploring shallower YOLO backbones or structured pruning to reduce Conv2D workload; (ii) evaluating resolution-accuracy trade-offs at 160×160 and 128×128; (iii) integrating the OV2640 camera for end-to-end pipeline testing; and (iv) empirical power measurement using an INA219 current sensor.

ACKNOWLEDGMENT

The authors thank the anonymous reviewers for their constructive feedback that improved the quality of this manuscript.

REFERENCES

[1] J. Redmon, S. Divvala, R. Girshick, and A. Farhadi, "You only look once: Unified, real-time object detection," in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. (CVPR), 2016, pp. 779–788.

[2] J. Redmon and A. Farhadi, "YOLOv3: An incremental improvement," arXiv preprint arXiv:1804.02767, 2018.

[3] A. Bochkovskiy, C.-Y. Wang, and H.-Y. M. Liao, "YOLOv4: Optimal speed and accuracy of object detection," arXiv preprint arXiv:2004.10934, 2020.

[4] G. Jocher, A. Chaurasia, and J. Qiu, "Ultralytics YOLO," GitHub, 2023. [Online]. Available: https://github.com/ultralytics/ultralytics

[5] P. Warden and D. Situnayake, TinyML: Machine Learning with TensorFlow Lite on Arduino and Ultra-Low-Power Microcontrollers. O’Reilly Media, 2019.

[6] J. Lin et al., "MCUNet: Tiny deep learning on IoT devices," in Proc. Adv. Neural Inf. Process. Syst. (NeurIPS), 2020.

[7] C. Banbury et al., "Benchmarking TinyML systems: Challenges and direction," arXiv preprint arXiv:2003.04821, 2020.

[8] L. Lai, N. Suda, and V. Chandra, "CMSIS-NN: Efficient neural network kernels for Arm Cortex-M CPUs," arXiv preprint arXiv:1801.06601, 2018.

[9] R. David et al., "TensorFlow Lite Micro: Embedded machine learning on TinyML systems," in Proc. Mach. Learn. Syst. (MLSys), 2021.

[10] C. Banbury et al., "MicroNets: Neural network architectures for deploying TinyML applications on commodity microcontrollers," in Proc. Mach. Learn. Syst. (MLSys), 2021.

[11] S. Xu et al., "Edge-YOLO: Real-time object detection on edge devices," arXiv preprint arXiv:2209.13083, 2022.

[12] B. Jacob et al., "Quantization and training of neural networks for efficient integer-arithmetic-only inference," in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. (CVPR), 2018, pp. 2704–2713.

[13] R. Krishnamoorthi, "Quantizing deep convolutional networks for efficient inference: A whitepaper," arXiv preprint arXiv:1806.08342, 2018.

[14] A. Gholami et al., "A survey of quantization methods for efficient neural network inference," arXiv preprint arXiv:2103.13630, 2021.

[15] S. Han, H. Mao, and W. J. Dally, "Deep compression: Compressing deep neural networks with pruning, trained quantization and Huffman coding," in Proc. Int. Conf. Learn. Represent. (ICLR), 2016.

[16] A. G. Howard et al., "MobileNets: Efficient convolutional neural networks for mobile vision applications," arXiv preprint arXiv:1704.04861, 2017.

[17] M. Sandler et al., "MobileNetV2: Inverted residuals and linear bottlenecks," in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. (CVPR), 2018, pp. 4510–4518.

[18] Espressif Systems, "ESP-NN: Optimised neural network functions for Espressif chips," GitHub, 2023. [Online]. Available: https://github.com/espressif/esp-nn

[19] T.-Y. Lin et al., "Microsoft COCO: Common objects in context," in Proc. Eur. Conf. Comput. Vis. (ECCV), 2014.

[20] N. Zhang et al., "Training a disaster victim detection network for UAV search and rescue using harmonious composite images," Remote Sens., vol. 14, no. 13, p. 2977, 2022.